# Analysis of the X-point Radiation in seeded plasmas in JET through tomography reconstruction

Ivan Wyss1, Valentina D’Agostino1,2, Pasquale Gaudio1, Michela Gelfusa1, Andrea Murari3,4, Novella Rutigliano1 , Tayyaba Sajid1 and Riccardo Rossi1 on behalf of JET contributors§ and EUROfusion Tokamak Exploitation Team£

1) Department of Industrial Engineering, “Tor Vergata” University of Rome, Via del Politecnico 1, 00133, Rome, Italy

2) ENEA, Nuclear Department, Via Enrico Fermi 45, 00044 Frascati, Italy

3) Consorzio RFX (CNR, ENEA, INFN, Università di Padova, Acciaierie Venete SpA), Corso Stati Uniti 4, 35127 Padova, Italy

4) Istituto per la Scienza e la Tecnologia dei Plasmi, CNR, Padova, Italy

§ See the author list of “Overview of T and D-T results in JET with ITER-like wall” by C.F. Maggi et al., to be published in Nuclear Fusion Special Issue from the 29th Fusion Energy Conference (London, UK, 2023)

£ See the author list of “Progress on an exhaust solution for a reactor using EUROfusion multi-machines capabilities” by E. Joffrin et al. to be published in Nuclear Fusion Special Issue: Overview and Summary Papers from the 29th Fusion Energy Conference (London, UK, 16-21 October 2023)

Corresponding author: ivan.wyss@ uniroma2.it

Mitigating the heat load to the divertor is a key challenge for future fusion reactors. Current material limits constrain the allowable heat flux to below 10 MW/m², requiring a significant fraction of the power exhausted from the core plasma to be radiated to maintain acceptable conditions at the divertor. In next-step devices, such as ITER and DEMO, sustained operation without damage demands that 60–75% of the total loss power in ITER (approximately 150 MW) and up to 95% in DEMO be radiated away. This can be achieved through impurity seeding, which promotes divertor detachment by forming a radiating region near the X-point that dissipates most of the exhaust energy. However, excessive impurity injection can cause core contamination and trigger disruptions. Understanding the stability and intensity of the radiation front is therefore essential for protecting the plasma and the machine. In this work, we propose a method to estimate both the position and intensity of the radiator, along with their uncertainties, using a maximum likelihood tomographic reconstruction technique. This approach enables the determination of the local radiated power and energy distribution around the X-point, supporting the analysis of radiator motion and MARFE formation. Furthermore, a first statistical study of impurity-seeded JET discharges is presented, identifying preliminary stability regions and potential local power-balance indicators for real-time control of the radiative regimes.