Plasma density diagnostics and machine-learning jitter analysis for stable PWFA operation at SPARC_LAB

R. Demitra1*, A. Biagioni1, L. Crincoli1, R. Pompili1 and M. Ferrario1.

1INFN-LNF , Via Enrico Fermi 54, 00044 Frascati (RM), Italy

*Corresponding author: romain.lwe@lnf.infn.it

Plasma-wakefield accelerators sustain GV/m-scale gradients, but shot-to-shot reproducibility remains the main obstacle to user-oriented operation. Diagnosing and controlling this instability requires measuring the plasma itself, not just the beam.

At the SPARC_LAB plasma-based accelerator facility at INFN-LNF, Stark broadening spectroscopy of the Hβ line provided shot-resolved plasma density measurements during live acceleration runs. Laser-triggered discharge operation reduced discharge fluctuations from ~10% to 1%, cutting empty shots from ~45% to 0.5% and witness-energy jitter by a factor of five.

Residual jitter was then studied on a dataset of ~941 shots across six working points and 19 machine features, combining Stark-derived density with upstream beam diagnostics. A heteroscedastic neural network quantifies predictive uncertainty shot-by-shot, while permutation importance and conditional mutual information isolate the contribution of each feature. Injected charge is the dominant driver of witness-energy fluctuations. Witness energy-spread jitter, by contrast, cannot be predicted from the measured machine state, setting a quantitative bound on what diagnostics and closed-loop controls can realistically achieve.